Library 1004: MDSV -- Digital Filter Design for the HP48GX

==========================================================

Martin A. Lang, October 1998

Short Description

-----------------

The MDSV library allows the design of digital filters (IIR lowpass and

highpass, FIR, differentiator, Hilbert transformer) and to plot the

frequency domain (magnitude and phase) and time domain (impulse and step)

responses of the designed filter or any other digital transfer function.

All routines are written in UserRPL. There wouldn't be much gain with SysRPL

as most of the time-consuming calculations must be done with complex

floating-point numbers.

The library has been tested on the HP48GX versions M and R. It does not run

on the HP48S(X).

The following documentation is kind of short, because the user interface is

self-explanatory, and the user is expected to know what this is all about.

Files in the archive MDSV.ZIP

-----------------------------

mdsv           library 1004 for HP48GX

mdsv.doc       this file

Use

---

Install MDSV in any port like any other library. It takes about 8K of RAM

and autoattaches to the HOME directory.

Invoke the library and select the MDSV menu item, which will present a menu

of options (scroll down to see all options) which is self-explanatory. The

last option is a short help text which explains some essential conventions

used in the library.

The plotting functions are slow because of the many floating point

operations. They beep when done, so this is a good opportunity to do

something else in between. After the first plot, press ON to see the second

one. The bottom text line in the plot designates the x-axes tick unit.

The library creates/uses a subdirectory DDSV in the current directory which

holds some variables and results (also explained in the short help text): HZ

and HN hold the numerator and denominator coefficients of the most recently

computed filter. Other coefficients may be stored there to plot other

transfer functions. SigmaDAT (�DAT) holds the most recently computed

frequency or time domain response with the same resolution as the

corresponding plot.

Algorithms

----------

The differentiator and the Hilbert transformer have been precomputed as FIR

filters, and the library just contains the filter coefficients.

FIR (finite impulse response) filters are designed with the Fourier method.

The frequency response magnitude is a user-defined wish function. If the

filter degree is user-defined, no time-domain window is used, otherwise the

wish function is subjected to a Kaiser-Bessel window.

IIR (infiniute impulse response) filters are designed as Tschebyscheff

(Tchebychev) I (ripple in passband, but not in stopband), using a bilinear

transformation.

All calculated coefficients are outputted as reals normalized to 1. This

means, the filters can be used "as is" on floating-point DSPs. For

fixed-point machines, the coefficients must be renormalized.

Notes

-----

Frequencies are normalized to the range 0..1, corresponding to 0..2*pi in

conventional digital signal processing notation.

Amplitudes are normalized to the range 0..1 and are absolute values (no dB

and such).

On the first invocation, all variables will be preset with some

meaningful value, so this is a good starting point for experiments or trying

out the functions.

Abbreviations used (most are explained in the user interface):

N       filter degree

HZ      numerator coefficients

HN      denominator coefficients

DP      passband ripple

DS      stopband ripple

SA      stopband amplitude

TW      transition width

FP      passband limit (end)frequency

FS      stopband limit (begin) frequency

Here's an opportunity to learn two German words that I used because they

are shorter than their English counterparts:

Zahler or Z�hler        numerator (of the transfer function)

Nenner                  denominator (of the transfer function)

Legalese

--------

1. The MDSV library is (C) 1998 Martin A. Lang

2. The MDSV library may not be sold and not be used in any other way to make

money with it.

3. Use the MDSV library at your own risk.

Author

------

Martin A. Lang

mlang@biotronik.de

������������������������������������� � �������������������������������������